APPENDIX A

Scenario Validation, Gap Consolidation and Architectural Refinement

This appendix documents the ATAM-inspired scenario validation used to evaluate baseline architecture. The validation was organized into four iterations by scenario criticality. Each scenario was mapped to requirements, layers, cross-cutting clusters and component traces. The analysis then assessed functional feasibility, structural conformance, latency, trade-offs, risks and refinement needs. The resulting gaps were consolidated to derive the refined architecture, which was subsequently re-validated.

Note: Relationship with the published baseline. The architecture evaluated here is the three-layer conceptual architecture presented in Figure 8 of the reference publication. That figure classifies architectural services as required, optional, or cross-cutting, and desc bribes it as a simplified conceptual model specifying logical responsibilities and interaction patterns rather than deployment. Five services are identified as cross-layers: Audit (C5), Gateway/PEP (C6), Trust Registry (C12), Verifier (C13) and FHIR Service (C15). Compliance (C24) is an optional Privacy & Policy component. Scenario-based validation operationalizes that logical model, and in doing so distinguishes cross-layer responsibility from per-layer availability. That distinction is the source of several gaps recorded here. These are not corrections to the baseline; they are the consequences of specifying, at scenario level, what a logical-level architecture necessarily leaves open, and they are precisely the operational validation the reference publication identifies as future work.

Section index

| Section | Content | Tables |
| --- | --- | --- |
| A.0 | Baseline architecture components and subcomponents | Table A-0 |
| A.1 | Scenario Overview Matrix | Table A-1 |
| A.2 | Functional Feasibility — Component Usage Matrix | Table A-2 |
| A.3 | Functional Feasibility — Gap Matrix | Table A-3 |
| A.4 | Structural Conformance — Layer Placement | Table A-4 |
| A.5 | Structural Conformance — Cross-Cutting Cluster Analysis | Table A-5 |
| A.6 | Latency Analysis — Component Benchmarks | Table A-6 |
| A.7 | Latency Analysis — Scenario Summary | Tables A-7, A-7.1 |
| A.8 | Trade-off Analysis — Consolidated | Table A-8 |
| A.9 | Risk Analysis — Risk Register | Table A-9 |
| A.10 | Gap Consolidation | Table A-10 |
| A.11 | Gap-to-Refinement Mapping | Tables A-11, A-11.1 to A-11.4 |
| A.12 | Refined Architecture | Table A-12 |
| A.13 | Gap Closure Verification | Table A-13 |
| A.14 | Re-Validation Summary | Tables A-14, A-15 |

# A.0  Baseline Architecture Components and Subcomponents

Table A-0: Baseline components, subcomponents and deployment

Layer placement follows Figure 8. Subcomponents are those exercised by the scenario traces in Appendix B. Capabilities marked as absent or not operationalised are the subject of the gaps recorded in Table A-3.

| Layer | Component | Subcomponents in the baseline | Deployment / gap |
| --- | --- | --- | --- |
| Access & Trust | C1 – Wallet / UI | C1.1 Web Wallet UI; C1.2 Mobile Wallet / Agent; C1.3 Local Credential Store; C1.4 OIDC4VC Client / Bridge | Required |
| Access & Trust | C7 – Identity Provider | C7.1 OIDC4VC Provider; C7.2 Token Issuer; C7.3 Authentication Module; C7.4 DID Resolver | Required |
| Privacy & Policy | C4 – Policy Engine (PDP) | C4.1 Policy Evaluator; C4.2 Policy Registry; C4.3 Decision API | Required. No obligation handler (C4.4) — G2 |
| Privacy & Policy | C14 – Issuer Service | C14.1 Consent VC Issuer; C14.2 Credential Signer; C14.3 Schema Binder; C14.4 Revocation Status | Optional |
| Privacy & Policy | C19 – Binding Service | C19.1 DID Resolver Hook; C19.2 DID–Patient Mapper; C19.3 Binding Store; C19.4 FHIR Patient.link Updater | Optional; binding is manual, not automated. No matching capability: candidate patient records cannot be retrieved or assessed prior to binding ---G14 |
| Privacy & Policy | C24 – Compliance Service | C24.5 Event Recorder; C24.6 Retention Manager; C24.7 Report Builder | Optional, and deployed in this layer only — G5 |
| Validation & Integration | C10 – Source Connectors | C10.1 Ingestion API; C10.2 Parser / Mapper; C10.3 Queue Buffer; C10.4 Retry Handler | Required. Single instance, no redundancy — G7. Subcomponents incomplete where invoked from Access — G8 |
| Validation & Integration | C17 – Standards Validator | C17.1 Profile Validator; C17.2 Terminology Validator; C17.3 Schema Checker; C17.4 Conformance Reporter | Required. No anonymiser (C17.5) — G11 |
| Validation & Integration | C23 – Queue / Event Bus | C23.1 Publisher; C23.2 Consumer; C23.3 Retry / Dead Letter Queue; C23.4 Metrics Exporter | Required |
| Validation & Integration | C25 – Orchestrator / Scheduler | C25.1 Retry Queue / DLQ; C25.2 Scheduler / Job Runner; C25.3 Backoff Controller; C25.4 Status Monitor; C25.5 Workflow Dispatcher | Required. No circuit breaker or degraded-mode controller — G13 |
| Cross-layer | C5 – Audit / Observability | C5.1 Event Logger; C5.2 Blockchain Anchor (optional); C5.3 Metrics Exporter; C5.4 Alert Hooks | All three layers. No structured tagging (C5.5) — G6 |
| Cross-layer | C6 – API Gateway / PEP | Access: C6.1 Auth Layer, C6.2 Policy Plugin Privacy: C6.3 Router, C6.4 Rate Limiter Integration: C6.5 Audit Hook, C6.7 Stream Guard, C6.8 Quota Controller | All three layers. C6.6 Data Minimizer specified as a Privacy & Policy responsibility but not enforced at any gateway instance — G3 |
| Cross-layer | C12 – Trust Registry / Resolver | C12.1 Issuer Registry; C12.2 Schema Store; C12.3 DID Resolver; C12.4 Credential Status / Revocation | Available to Access & Trust and Validation & Integration. Not available where consent is evaluated — G4. No cache (C12.5) — G9 |
| Cross-layer | C13 – Verifier Service | Access: C13.1 Proof Validator, C13.2 Signature Verifier Privacy: C13.3 Credential Parser, C13.4 VC Policy Evaluator Integration: C13.5–8 batch verification | All three layers |
| Cross-layer | C15 – FHIR Server | C15.1 REST API; C15.2 Database Layer; C15.3 Bundle Normalizer; C15.4 Authorization Filter | All three layers, but not fully operationalized in the integration and recovery path — G1 |

Component numbering. The baseline contains fifteen components, numbered non-contiguously (C1, C4, C5, C6 , C7, C12, C13, C14, C15, C17, C19, C23, C24, C25). The gaps in the sequence arise from the consolidation described in the reference publication, in which redundant components were merged into core services. C26 and C27 are introduced by the refinement in Section A.12.

The behavioral models for R2 and R7 refer to a “Consent Manager” queried by the gateway to establish whether a valid consent grant exists. This is a role rather than a component: it is realized by C14 Issuer Service, which issues and maintains consent credentials and their revocation status, together with C4.2 Policy Registry, which holds current consent state and is read during policy evaluation. The scenario traces in Appendix B use the component identifiers throughout.

# A.1 Scenario Overview Matrix

Table A-1: Scenario overview

The scenario set was not fixed at a predetermined number. Scenarios were generated from the requirement set and from the runtime flows in the reference publication (A Modular Three Layered Architecture for Blockchain-Enabled Self-Sovereign Identity in Health Information Exchange | International Journal of Information Security | Springer Nature Link), then organized into four iterations by criticality, until each requirement was represented and each layer, cross-cutting service and quality attribute was exercised at least once. Fifteen scenarios satisfied those conditions.

The set was constructed to span condition classes rather than to reach a particular size: emergency override, consent-mediated access, cross-organizational exchange, bulk ingestion, failure during operation, authorization change mid-episode, secondary use, retrospective investigation, and multi-device access. Section A.16.3 shows why this matters: three of the thirteen gaps surface under almost any scenario, while eight are each confined to a single condition class.

Cross-cutting clusters are drawn from the five cross-layer services in Figure 8: C5, C6, C12, C13, C15.

| ID | Scenario Name | Priority | Primary Layer | Secondary | Clusters | Requirements |
| --- | --- | --- | --- | --- | --- | --- |
| S1 | Emergency Physician Access | HIGH | Access & Trust; Privacy & Policy | Integration | C5, C6, C12, C13, C15 | R1, R7, R8, R11, R14 |
| S2 | Consent-Based Specialist Access | HIGH | Privacy & Policy | Access & Trust | C5, C6, C13, C15 | R2, R6, R7, R8, R11, R13 |
| S3 | Laboratory Ingestion with Recovery | HIGH | Validation & Integration | Access; Privacy | C5, C6, C12, C13, C15 | R3, R4, R9, R10, R12, R13, R16 |
| S4 | Cross-Organizational Trust Federation | HIGH | Access & Trust; Privacy & Policy | Integration | C5, C6, C12, C13, C15 | R4, R11, R12, R13 |
| S5 | Patient-Controlled Research Donation | MEDIUM | Privacy & Policy | Integration | C5, C6, C12, C13 | R2, R8, R9, R10, R11, R16 |
| S6 | Emergency Override with Escalation | HIGH | Privacy & Policy | Access & Trust | C5, C6, C12, C13, C15 | R7, R8, R11, R16 |
| S7 | Mass Data Ingestion Peak Load | HIGH | Validation & Integration | – | C5, C6, C12, C13, C15 | R3, R9, R10, R16 |
| S8 | Provider Role Change During Session | MEDIUM | Access & Trust | Privacy & Policy | C5, C6, C12, C13, C15 | R1, R7, R12, R15 |
| S9 | Multi-Device Patient Access | LOW | Access & Trust | Privacy & Policy | C5, C6, C12, C13, C15 | R1, R2, R14, R15 |
| S10 | Regulatory Audit Investigation | HIGH | Access & Trust; Privacy & Policy | – | C5, C6 | R11, R16 |
| S11 | Patient Registration with Identity Binding | MEDIUM | Access & Trust | Privacy & Policy | C5, C15 | R1, R5, R6, R12, R13 |
| S12 | Consent Revocation During Active Access | HIGH | Privacy & Policy | Access & Trust | C5, C6, C13, C15 | R2, R7, R8, R11 |
| S13 | Pharmacy Prescription Fulfilment | MEDIUM | Access & Trust; Privacy & Policy | Integration | C5, C6, C12, C13, C15 | R1, R4, R5, R7, R13 |
| S14 | Trust Resolution Failure During Node Outage | HIGH | Access & Trust | Integration | C5, C6, C12, C13 | R4, R5, R16 |
| S15 | IoT Device Continuous Data Stream | MEDIUM | Validation & Integration | – | C5, C6, C15 | R3, R9, R10, R16 |

# A.2 Functional Feasibility — Component Usage Matrix

Table A-2: Component usage across all scenarios

| Symbol | Meaning |
| --- | --- |
| ✓ | Component appears as an executing step in the scenario's component trace |
| ✗ | Component is required by the scenario but unavailable in the baseline — recorded as a gap, not a usage |
| – | Not involved in this scenario |

Part 1 of 2 — scenarios S1 to S8

| Component | S1 | S2 | S3 | S4 | S5 | S6 | S7 | S8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C1 – Wallet | ✓ | ✓ | – | ✓ | – | ✓ | – | ✓ |
| C4 – Policy Engine | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | – | ✓ |
| C5 – Audit | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| C6 – PEP | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| C7 – IdP | ✓ | ✓ | ✓ | ✓ | – | ✓ | – | ✓ |
| C10 – Connector | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| C12 – Trust Registry | ✓ | ✗ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| C13 – Verifier | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| C14 – Issuer | – | – | – | – | – | – | – | – |
| C15 – FHIR Server | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| C17 – Validator | – | – | ✓ | – | ✓ | – | ✓ | – |
| C19 – Binding | ✓ | ✓ | ✓ | – | – | – | – | – |
| C23 – Queue | – | – | ✓ | – | ✓ | – | ✓ | – |
| C24 – Compliance | ✓ | ✗ | ✓ | ✓ | ✓ | ✓ | ✓ | – |
| C25 – Orchestrator | – | – | ✓ | – | ✓ | – | ✓ | – |

Part 2 of 2 — scenarios S9 to S15, with totals

| Component | S9 | S10 | S11 | S12 | S13 | S14 | S15 | Used | Gap ✗ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C1 – Wallet | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | – | 11 | 0 |
| C4 – Policy Engine | ✓ | ✓ | – | ✓ | ✓ | – | – | 11 | 0 |
| C5 – Audit | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | 15 | 0 |
| C6 – PEP | ✓ | ✓ | – | ✓ | ✓ | ✓ | ✓ | 14 | 0 |
| C7 – IdP | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | – | 12 | 0 |
| C10 – Connector | – | – | – | ✓ | ✓ | – | ✓ | 11 | 0 |
| C12 – Trust Registry | ✓ | – | – | ✗ | ✓ | ✓ | – | 10 | 2 |
| C13 – Verifier | ✓ | ✓ | – | ✓ | ✓ | ✓ | – | 13 | 0 |
| C14 – Issuer | ✓ | – | ✓ | ✓ | – | – | – | 3 | 0 |
| C15 – FHIR Server | ✓ | – | ✓ | ✓ | ✓ | – | ✓ | 13 | 0 |
| C17 – Validator | – | – | – | – | – | – | ✓ | 4 | 0 |
| C19 – Binding | ✓ | – | ✓ | ✓ | – | – | – | 6 | 0 |
| C23 – Queue | – | – | – | – | – | – | ✓ | 4 | 0 |
| C24 – Compliance | – | ✗ | – | ✗ | – | ✗ | ✓ | 7 | 4 |
| C25 – Orchestrator | – | – | – | – | – | – | ✓ | 4 | 0 |

# A.3  Functional Feasibility — Gap Matrix

Table A-3: Gaps identified in the baseline architecture

| ID | Gap | Component / missing element | Affected scenarios | No. | Severity |
| --- | --- | --- | --- | --- | --- |
| G1 | FHIR services only partially operationalised in Integration | C15 present in the integration path without full storage (C15.2) and authorization-filter (C15.4) capability | S3, S7 | 2 | Critical |
| G2 | No obligation handling | C4.4 Obligation Handler absent | S1, S2, S4, S5, S6, S10, S12, S13 | 8 | Critical |
| G3 | Data minimisation specified but not operationalised | C6.6 Data Minimizer defined as a Privacy & Policy responsibility, not enforced at any gateway instance | S1, S2, S4, S6, S10 | 5 | Critical |
| G4 | Trust verification unavailable where consent is evaluated | C12 not available to the Privacy & Policy cluster | S2, S12 | 2 | Critical |
| G5 | Compliance recording optional and single-layer | C24 optional and deployed only in Privacy & Policy | S1, S2, S3, S4, S5, S6, S7, S10, S12, S14, S15 | 11 | Critical |
| G6 | No structured audit tagging | C5.5 Audit Tagging absent | All fifteen scenarios | 15 | Medium |
| G7 | Connector deployed as a single instance | C10.5–6 redundant instances absent | S1, S3, S7, S15 | 4 | Critical |
| G8 | Connector subcomponents incomplete in Access | C10.2 parser and C10.3 buffer not available to Access-layer flows | S4 | 1 | Medium |
| G9 | No trust cache or fallback trust resolution | C12.5 Trust Cache absent | S1, S3, S4, S5, S6, S7, S8, S9, S13, S14 | 10 | Critical |
| G10 | No session management | C27 Session Manager absent | S6, S8, S9, S12 | 4 | Medium |
| G11 | No anonymisation capability | C26 Anonymizer and C17.5 absent | S5 | 1 | Critical |
| G12 | No cross-device session and consent synchronisation | C27.2 Session Sync absent | S9 | 1 | Medium |
| G13 | No degraded-mode operation or retry policy | C25.6 Circuit Breaker and C25.7 Degraded-Mode Controller absent | S3, S14 | 2 | Critical |
| G14 | No patient identity resolution | C19.5 Identity Matcher absent; no component retrieves or assesses candidate patient records | S11 (Not exercised) | 1 | Critical |

G6 is universal. Structured audit tagging is absent in all fifteen scenarios — every trace produces audit records that cannot be filtered by event class, subject, device or outcome. This is the only gap with complete coverage across the scenario set, and it is the single most broadly evidenced finding in the validation.

# A.4 Structural Conformance — Layer Placement

Table A-4: Layer placement findings

| Component | Baseline placement (Fig. 8) | Required by the scenarios | Demonstrated in | Gap |
| --- | --- | --- | --- | --- |
| C12 | Cross-layer, available to Access & Trust and Validation & Integration | Also required where consent is evaluated | S2, S12 | G4 |
| C15 | Cross-layer, all three layers | Full storage and authorization-filter capability within the integration path | S3, S7 | G1 |
| C24 | Privacy & Policy, optional | Required in all three layers and mandatory | S1, S2, S3, S4, S5, S6, S7, S10, S12, S14, S15 | G5 |
| C6.6 | Privacy & Policy responsibility | Enforced at every gateway instance | S1, S2, S4, S6, S10 | G3 |
| C10 | Validation & Integration | Also invoked from Access-layer flows, with full subcomponents | S4 | G8 |
| C19 | Privacy and Policy | Matching before binding (R6 model) | S11 | G14 |

# A.5 Structural Conformance — Cross-Cutting Cluster Analysis

Table A-5: Cross-cutting cluster participation

| Cluster | Layer availability in the baseline | Issues found | Affected scenarios |
| --- | --- | --- | --- |
| C5 – Audit | All three layers (C5.1–4) | No structured tagging (G6) | All fifteen |
| C6 – PEP | All three layers | C6.6 not enforced at any instance (G3) | S1, S2, S4, S6, S10 |
| C12 – Trust Registry | Access & Trust; Validation & Integration | Unavailable to the Privacy cluster (G4); no cache (G9) | G4: S2, S12 G9: S1, S3, S4, S5, S6, S7, S8, S9, S13, S14 |
| C13 – Verifier | All three layers (C13.1–8) | None recorded | – |
| C15 – FHIR Server | All three layers | Not fully operationalised in the integration path (G1) | S3, S7 |

# A.6 Latency Analysis — Component Benchmarks

Table A-6: Component latencies

| Component | Base Latency | Margin | Note |
| --- | --- | --- | --- |
| C1 – Wallet | 5 ms | 20% | – |
| C4 – Policy Engine | 1–10 ms | 20% | – |
| C5 – Audit (write) | 1–5 ms | 20% | – |
| C5 – Audit (query) | ≈100 ms | – | Estimate; no benchmark. See S10 |
| C6 – PEP | 2–5 ms | 20% | – |
| C7 – IdP | 10–20 ms | 20% | – |
| C10 – Connector | 2–10 ms | 50% | – |
| C12 – Trust Registry (on-chain) | 2000–5000 ms | 30% | 5000 ms also serves as the timeout threshold in S14 |
| C13 – Verifier | 10–50 ms | 20% | – |
| C14 – Issuer | 10–30 ms | 20% | – |
| C15 – FHIR Server | 0.6–5 ms | 50% | – |
| C17 – Validator | 5–10 ms | 20% | – |
| C19 – Binding | 5–15 ms | 20% | – |
| C23 – Queue (publish/consume) | 2–5 ms | 20% | Steady-state message handling |
| C23 – Queue (buffer for retry) | 15–25 ms | 20% | Durable write plus retry-state registration. See S3 |
| C24 – Compliance | 3 ms | 20% | Lightweight, off-chain |
| C25 – Orchestrator | 5–20 ms | 20% | – |
| Network | 2–5 ms/hop | – | – |

# A.7  Latency Analysis — Scenario Summary

Table A-7: Latency by scenario, derived from the Appendix B component traces

| Scen. | On-chain | Off-chain | Network | Total | Bottleneck / result |
| --- | --- | --- | --- | --- | --- |
| S1 | 4550 ms | 88.95 ms | 9 ms | 4647.95 ms | C12 (97.9%). Target <5 s met, 352 ms margin |
| S2 | 0 ms | 109.35 ms | – | 109.35 ms | No on-chain path. Fastest scenario |
| S3 | 4550 ms | 128.55 ms | 15 ms | 4693.55 ms | C12 (96.9%) |
| S4 | 4550 ms | 82.95 ms | 9 ms | 4641.95 ms | C12 (98.0%). Target <5 s met, 358 ms margin |
| S5 | ≈ 4.21 hr | 195.35 s | – | ≈ 4.27 hr | C12 ×10k (99.9%). Target <1 hr NOT met |
| S6 | 9100 ms | 153.75 ms | – | 9253.75 ms | C12 ×2 (98.3%). Escalation doubles the lookup |
| S7 | ≈ 4.20 hr | 302.6 s | – | ≈ 4.30 hr | C12 ×10k (99.9%). Throughput target NOT met |
| S8 | 4550 ms | 104.55 ms | – | 4654.55 ms | C12 (97.8%). Enforcement lag unbounded |
| S9 | 13,650 ms | 269.55 ms | 36 ms | 13,955.55 ms | C12 ×3 (97.8%). Cost scales with device count |
| S10 | 0 ms | 30,170.8 ms machine 15,000 ms human | – | ≈ 45.2 s | Full audit scan. Target <5 s NOT met |
| S11 | 0 ms | 136.7 ms | – | 136.7 ms | Target <200 ms met |
| S12 | 0 ms | 158.55 ms | – | 158.55 ms | Revocation enforcement not time-bound |
| S13 | 4550 ms | 93.75 ms | – | 4643.75 ms | C12 (98.0%). Target <5 s met, 356 ms margin |
| S14 | 5000 ms (timeout) | 63.6 ms | – | 5063.6 ms | C12 timeout (98.7%). Access DENIED |
| S15 | 0 ms | 41.85 ms/msg | – | 41.85 ms/msg | Target <50 ms met, 8.15 ms margin |

Basis notes. S5 and S7 are batch scenarios; figures are effective elapsed time for 10,000 records with the stated parallelism. S10 measures an investigation workflow, so machine and human effort are reported separately and must not be summed into a figure comparable with the other scenarios. S15 reports per-message latency at 100 messages per second. Network cost is included only where the corresponding trace models it explicitly.

Table A-7.1: Latency improvement with trust caching

Applying the 450 ms cached average from Table A-8 to each on-chain lookup. Covers all ten scenarios affected by G9.

| Scenario | Baseline | With cache | Improvement | Lookups |
| --- | --- | --- | --- | --- |
| S1 | 4647.95 ms | 547.95 ms | ~8.5× | 1 |
| S3 | 4693.55 ms | 593.55 ms | ~7.9× | 1 |
| S4 | 4641.95 ms | 541.95 ms | ~8.6× | 1 |
| S5 | ≈ 4.27 hr | ≈ 28.3 min | ~9.1× | 10,000 |
| S6 | 9253.75 ms | 1053.75 ms | ~8.8× | 2 |
| S7 | ≈ 4.30 hr | ≈ 30.0 min | ~8.6× | 10,000 |
| S8 | 4654.55 ms | 554.55 ms | ~8.4× | 1 |
| S9 | 13,955.55 ms | 1655.55 ms | ~8.4× | 3 |
| S13 | 4643.75 ms | 543.75 ms | ~8.5× | 1 |
| S14 | Access denied (timeout) | Access granted from cache | Not a ratio | 1 (failed) |

Assumption. The 450 ms average assumes a 90% cache hit rate. That assumption carries the whole table: at a 50% hit rate the average would be approximately 2500 ms and every improvement factor would fall below 2×. The refined C12.5 must therefore specify both a freshness bound and a revocation-propagation rule, or the performance gain is purchased at the cost of accepting revoked credentials. S3 shows the lowest factor because it carries the most off-chain work; S14 cannot be expressed as a ratio because its baseline is a failure rather than a latency.

# A.8 Trade-off Analysis — Consolidated

Table A-8: Recurring trade-offs across scenarios

| ID | Trade-off | Appears in | Components | Decision |
| --- | --- | --- | --- | --- |
| T1 | Security vs performance in trust resolution | S1, S3, S4, S5, S6, S7, S8, S9, S13, S14 | C12 | Add C12.5 cache (90% hit, 450 ms average) with an explicit freshness bound |
| T2 | Privacy vs simplicity in policy enforcement | S1, S2, S4, S6, S10 | C4, C6 | Add C4.4 obligation handling; operationalise C6.6 at every gateway instance |
| T3 | Consistency vs availability in ingestion | S3, S7, S15 | C23, C25 | Asynchronous processing acceptable, but bounded by a retry policy and circuit breaker |
| T4 | Completeness vs flexibility in governance | S1–S7, S10, S12, S14, S15 | C24 | Make C24 required and deploy across all three layers |
| T5 | Redundancy vs cost at the ingestion boundary | S1, S3, S7, S15 | C10 | Add C10.5–6 redundant instances |
| T6 | Statelessness vs enforcement immediacy | S6, S8, S9, S12 | C27 (new) | Add session correlation and change signalling without per-request server state |
| T7 | Write simplicity vs read capability in audit | All fifteen | C5 | Add C5.5 tagging; move the cost to write time where it is affordable |

# A.9 Risk Analysis — Risk Register

Risk identifiers use the RK prefix to avoid collision with the requirement identifiers R1–R16 used throughout the scenario tables.

Table A-9: Risk register

| ID | Description | Affected scenarios | Likelihood | Impact | Gap |
| --- | --- | --- | --- | --- | --- |
| RK1 | FHIR services not fully operationalised in the integration path | S3, S7 | High | Critical | G1 |
| RK2 | Obligations cannot be executed | S1, S2, S4, S5, S6, S10, S12, S13 | High | Critical | G2 |
| RK3 | Data minimisation not enforced | S1, S2, S4, S6, S10 | High | Critical | G3 |
| RK4 | Consent-issuer trust cannot be established | S2, S12 | Medium | Critical | G4 |
| RK5 | Compliance events unrecorded | S1–S7, S10, S12, S14, S15 | High | Critical | G5 |
| RK6 | Audit records not retrievable by event class | All fifteen | High | Medium | G6 |
| RK7 | Ingestion bottleneck and no connector failover | S1, S3, S7, S15 | Medium | Critical | G7 |
| RK8 | Access-layer integration capability incomplete | S4 | Medium | Medium | G8 |
| RK9 | Trust resolution dominates latency and fails on outage | S1, S3, S4, S5, S6, S7, S8, S9, S13, S14 | High | Critical | G9 |
| RK10 | Authorisation changes not enforced on active sessions | S6, S8, S9, S12 | Medium | Medium | G10 |
| RK11 | PII not removed from research datasets | S5 | Medium | Critical | G11 |
| RK12 | Consent state diverges across patient devices | S9 | Low | Medium | G12 |
| RK13 | Transient failures indistinguishable from permanent ones | S3, S14 | Medium | Critical | G13 |
| RK14 | DID bound to an incorrect patient record | S11 | Medium | Critical | G14 |

# A.10  Gap Consolidation

Table A-10: Consolidated gap register

Thirteen gaps were identified across the fifteen scenarios, giving 66 gap instances in total.

| ID | Gap | Count | Severity | Derived from | Closed by |
| --- | --- | --- | --- | --- | --- |
| G1 | FHIR services only partially operationalised in Integration | 2 | Critical | T3, RK1 | C15.2, C15.4 completion |
| G2 | No obligation handling | 8 | Critical | T2, RK2 | C4.4 |
| G3 | Data minimisation specified but not operationalised | 5 | Critical | T2, RK3 | C6.6 enforcement |
| G4 | Trust verification unavailable where consent is evaluated | 2 | Critical | T1, RK4 | C12 in Privacy |
| G5 | Compliance recording optional and single-layer | 11 | Critical | T4, RK5 | C24 required, all layers |
| G6 | No structured audit tagging | 15 | Medium | T7, RK6 | C5.5 |
| G7 | Connector deployed as a single instance | 4 | Critical | T5, RK7 | C10.5–6 |
| G8 | Connector subcomponents incomplete in Access | 1 | Medium | T5, RK8 | C10 full in Access |
| G9 | No trust cache or fallback trust resolution | 10 | Critical | T1, RK9 | C12.5 |
| G10 | No session management | 4 | Medium | T6, RK10 | C27 |
| G11 | No anonymisation capability | 1 | Critical | T4, RK11 | C26 + C17.5 |
| G12 | No cross-device session and consent synchronisation | 1 | Medium | T6, RK12 | C27.2 |
| G13 | No degraded-mode operation or retry policy | 2 | Critical | T3, RK13 | C25.6, C25.7 |
| G14 | No patient identity resolutions | 1 | Critical | T2, RK14 | C19.5 |

# A.11  Gap-to-Refinement Mapping

The tables in this section describe the refined architecture. Component placements shown here are the target state after refinement and differ from the baseline placements in Table A-0.

Table A-11: Refinements for all gaps

| Gap | Refinement | Component | Type |
| --- | --- | --- | --- |
| G1 | Complete C15 in the integration path by adding C15.2 storage and C15.4 authorization filter | C15 | Completion of deployment |
| G2 | Add C4.4 Obligation Handler | C4 | New subcomponent |
| G3 | Operationalise C6.6 Data Minimizer at every gateway instance | C6 | Deployment of specified capability |
| G4 | Make C12 available to the Privacy & Policy cluster | C12 | Extended availability |
| G5 | Make C24 required and deploy across all three layers | C24 | Type change and extension |
| G6 | Add C5.5 Audit Tagging | C5 | New subcomponent |
| G7 | Add C10.5–6 redundant connector instances | C10 | New instances |
| G8 | Make all C10 subcomponents available to Access-layer flows | C10 | Requirement change |
| G9 | Add C12.5 Trust Cache with a bounded freshness window and revocation propagation | C12 | New subcomponent |
| G10 | Add C27 Session Manager | C27 (new) | New component |
| G11 | Add C26 Anonymizer and C17.5 PII Verifier | C26 (new) + C17 | New component and subcomponent |
| G12 | Add C27.2 Cross-Device Session Synchronisation | C27 | New subcomponent |
| G13 | Add C25.6 Circuit Breaker and C25.7 Degraded-Mode Controller | C25 | New subcomponents |
| G14 | Add C19.5 Identity Matcher — retrieves candidate records and applies deterministic or probabilistic matching before binding | C19 | New subcomponent |

Table A-11.1: Refined layer responsibilities

| Layer | Baseline responsibility | Refined responsibility | Gaps addressed |
| --- | --- | --- | --- |
| Access & Trust | Identity establishment and credential verification | Plus session management and cross-device state synchronisation (C27) | G10, G12 |
| Privacy & Policy | Consent validation and access decisions | Plus obligation execution (C4.4), trust availability (C12), mandatory compliance (C24), enforced minimisation (C6.6) | G2, G3, G4, G5 |
| Validation & Integration | Connectivity, validation and recovery | Plus complete FHIR capability (C15.2, C15.4), connector redundancy (C10.5–6), bounded recovery (C25.6–7), anonymisation (C26) | G1, G7, G11, G13 |
| Cross-layer | Audit, gateway, trust, verification, FHIR | Plus audit tagging (C5.5), trust caching (C12.5), and C24 promoted to cross-layer | G5, G6, G9 |

Table A-11.2: Component changes

| Component | Change | From | To | Gap |
| --- | --- | --- | --- | --- |
| C4 | Add subcomponent | C4.1–3 | C4.1–4 (+C4.4) | G2 |
| C5 | Add subcomponent | C5.1–4 | C5.1–5 (+C5.5) | G6 |
| C6 | Deploy specified capability | C6.6 specified, not enforced | C6.6 enforced at all instances | G3 |
| C10 | Add instances | Single instance | C10.5–6 redundant instances | G7 |
| C10 | Extend availability | Integration only | Full subcomponents in Access | G8 |
| C12 | Add subcomponent | C12.1–4 | C12.1–5 (+C12.5 cache) | G9 |
| C12 | Extend availability | Access, Integration | Add Privacy & Policy | G4 |
| C15 | Complete deployment | Partial in Integration | C15.2 and C15.4 added | G1 |
| C17 | Add subcomponent | C17.1–4 | C17.1–5 (+C17.5) | G11 |
| C24 | Change type and extend | Optional, Privacy only | Required, all three layers | G5 |
| C25 | Add subcomponents | C25.1–5 | C25.1–7 (+C25.6, C25.7) | G13 |
| C26 | New component | – | Integration required; Privacy optional | G11 |
| C27 | New component | – | Access & Trust | G10, G12 |

Table A-11.3: Dependency analysis (refined)

| Source Layer | Target Layer | Allowed | Enforcement |
| --- | --- | --- | --- |
| Access & Trust | Privacy & Policy | Yes | Via C6 PEP |
| Access & Trust | Validation & Integration | Yes | Via C6 PEP |
| Privacy & Policy | Access & Trust | No | Blocked for data flow |
| Privacy & Policy | Validation & Integration | Yes | Via C6 PEP |
| Validation & Integration | Access & Trust | No | Blocked for data flow |
| Validation & Integration | Privacy & Policy | Yes | Via C6 PEP |
| Any layer | Cross-layer services (C5, C6, C12, C13, C15, C24) | Yes | Direct — cross-layer services are available from every layer and are not subject to the directional restriction |

The final row resolves an apparent conflict: S3, S5 and S7 are Integration-primary scenarios that invoke C7, C12 and C13. The directional restriction governs data flow between layer-specific components, not access to cross-layer services.

Table A-11.4: Security boundary analysis (refined)

| Boundary | Components | Trust Level | Authentication | Authorization |
| --- | --- | --- | --- | --- |
| A: Client Edge | C1 | Low | User credentials | None |
| B: Access Layer | C7, C13.1-2, C12, C27, C6.1-2, C5, C24 | Medium | OIDC4VC | Role-based |
| C: Privacy Layer | C4, C14, C19, C13.3-4, C6.3-4, C12, C24, C5, C26 | High | Verified plus consent | Policy-based |
| D: Integration Layer | C10, C17, C23, C25, C13.5-8, C6.5-8, C15, C26, C5, C24 | Medium | System credentials | Service accounts |
| E: External | External FHIR endpoints | Variable | OAuth2 or API keys | Resource-based |
| F: Cross-layer | C5, C6, C12, C13, C15, C24 | High | Internal | Service-to-service |

# A.12  Refined Architecture

Table A-12: Refined component distribution

| Component | Access | Privacy | Integration | Type | Change |
| --- | --- | --- | --- | --- | --- |
| C1 – Wallet | ✓ | – | – | Required | – |
| C4 – Policy Engine | – | ✓ (+C4.4) | – | Required | +C4.4 |
| C5 – Audit | ✓ (+C5.5) | ✓ (+C5.5) | ✓ (+C5.5) | Cross-layer | +C5.5 |
| C6 – PEP | ✓ (C6.6) | ✓ (C6.6) | ✓ (C6.6) | Cross-layer | C6.6 enforced |
| C7 – IdP | ✓ | – | – | Required | – |
| C10 – Connector | ✓ (full) | – | ✓ (+C10.5–6) | Required | +Access, +redundancy |
| C12 – Trust Registry | ✓ (+C12.5) | ✓ (+C12.5) | ✓ (+C12.5) | Cross-layer | +Privacy, +C12.5 |
| C13 – Verifier | ✓ | ✓ | ✓ | Cross-layer | – |
| C14 – Issuer | – | Optional | – | Optional | – |
| C15 – FHIR Server | ✓ | ✓ | ✓ (+C15.2, C15.4) | Cross-layer | Completed in Integration |
| C17 – Validator | – | – | ✓ (+C17.5) | Required | +C17.5 |
| C19 – Binding | – | ✓ | ✓ (+C19.5) | Optional -> Required | Made required |
| C23 – Queue | – | – | ✓ | Required | – |
| C24 – Compliance | ✓ (req) | ✓ (req) | ✓ (req) | Cross-layer | +Required, +Access, +Integration |
| C25 – Orchestrator | – | – | ✓ (+C25.6–7) | Required | +C25.6, +C25.7 |
| C26 – Anonymizer (NEW) | – | Optional | ✓ (req) | Req / Opt | NEW |
| C27 – Session Manager (NEW) | ✓ (+C27.2) | – | – | Required | NEW |

Cross-layer services increase from five to six. The baseline identifies C5, C6, C12, C13 and C15 as cross-layer. The refinement promotes C24 Compliance to the same status, since G5 is demonstrated in eleven of the fifteen scenarios and arises specifically from C24 being confined to one layer and optional within it.

# A.13  Gap Closure Verification

Table A-13: Gap closure

| Gap | Status | Verification |
| --- | --- | --- |
| G1 | CLOSED | C15.2 Database Layer and C15.4 Authorization Filter added to the integration path, enabling end-to-end ingestion with authorization applied at the write boundary |
| G2 | CLOSED | C4.4 Obligation Handler added, allowing notifications, masking, escalation and time-bound constraints to be attached to a policy decision |
| G3 | CLOSED | C6.6 Data Minimizer enforced at every gateway instance, so minimisation applies wherever a request is intercepted rather than only where it is specified |
| G4 | CLOSED | C12 made available to the Privacy & Policy cluster, so consent-issuer trust can be established where consent is evaluated |
| G5 | CLOSED | C24 made required and deployed across all three layers; promoted to a cross-layer service |
| G6 | CLOSED | C5.5 Audit Tagging added, carrying event class, subject, device, session and consent-version context; converts full-store scanning into targeted retrieval |
| G7 | CLOSED | C10.5–6 redundant connector instances added, providing failover and concurrent ingestion capacity |
| G8 | CLOSED | All C10 subcomponents made available to Access-layer flows, completing parser and buffer capability for cross-organisational exchange |
| G9 | CLOSED | C12.5 Trust Cache added with an explicit freshness bound and revocation-propagation rule. Reduces trust resolution to a 450 ms average at a 90% hit rate |
| G10 | CLOSED | C27 Session Manager added, providing request correlation and active change signalling without per-request server-side state |
| G11 | CLOSED | C26 Anonymizer added to perform PII removal, with C17.5 verifying that output contains no residual identifiers |
| G12 | CLOSED | C27.2 Cross-Device Session Synchronisation added, propagating consent and session changes to all active devices |
| G13 | CLOSED | C25.6 Circuit Breaker and C25.7 Degraded-Mode Controller added, bounding retry attempts, distinguishing transient from permanent failure, and raising an operator signal |
| G14 | CLOSED | C19.5 Identity Matcher added, retrieving candidate patient records and applying deterministic or probabilistic matching before a binding is created, preventing a decentralised identifier from being bound to an incorrect record. |

# A.14  Re-Validation Summary

Table A-14: Scenario status before and after refinement

| Scenario | Baseline | Refined | Gaps closed |
| --- | --- | --- | --- |
| S1 – Emergency Access | Partial | PASS | G2, G3, G5, G6, G7, G9 |
| S2 – Consent Access | Partial | PASS | G2, G3, G4, G5, G6 |
| S3 – Lab Ingestion | Partial | PASS | G1, G5, G6, G7, G9, G13 |
| S4 – Cross-Org Federation | Partial | PASS | G2, G3, G5, G6, G8, G9 |
| S5 – Research Donation | Fail | PASS | G2, G5, G6, G9, G11 |
| S6 – Escalation | Partial | PASS | G2, G3, G5, G6, G9, G10 |
| S7 – Peak Load | Fail | PASS | G1, G5, G6, G7, G9 |
| S8 – Role Change | Partial | PASS | G6, G9, G10 |
| S9 – Multi-Device | Partial | PASS | G6, G9, G10, G12 |
| S10 – Regulatory Audit | Fail | PASS | G2, G3, G5, G6 |
| S11 – Registration | Partial | PASS | G6, G14 |
| S12 – Revocation | Partial | PASS | G2, G4, G5, G6, G10 |
| S13 – Pharmacy | Partial | PASS | G2, G6, G9 |
| S14 – Node Outage | Fail | PASS | G5, G6, G9, G13 |
| S15 – IoT Stream | Partial | PASS | G5, G6, G7 |

No scenario passes against the baseline. Eleven are partial and four fail outright — S5 and S7 on throughput, S10 on retrieval, S14 on availability. All fifteen pass after refinement.

Refined component nature

In the refined architecture the nature of each component becomes explicit. Required cross-layer services comprise C5 Audit, C6 Gateway/PEP, C12 Trust Registry, C13 Verifier, C15 FHIR Server and C24 Compliance, all consistently deployed across the three layers. Required layer-specific components comprise C1 Wallet, C4 Policy Engine, C7 Identity Provider, C10 Connector, C17 Validator, C19 Binding Service, C23 Queue, C25 Orchestrator and C27 Session Manager. Optional or scenario-dependent components comprise C14 Issuer and C26 Anonymizer, the latter required only for workflows involving anonymisation or research-data processing.

Table A-15: Component nature in the refined architecture

| Component | Refined nature | Deployment character | Change from baseline |
| --- | --- | --- | --- |
| C1 | Required | Layer-specific (Access) | – |
| C4 | Required | Layer-specific (Privacy) | +C4.4 |
| C5 | Required | Cross-layer | +C5.5 |
| C6 | Required | Cross-layer | C6.6 enforced |
| C7 | Required | Layer-specific (Access) | – |
| C10 | Required | Layer-specific (Access, Integration) | +Access, +redundancy |
| C12 | Required | Cross-layer | +Privacy, +C12.5 |
| C13 | Required | Cross-layer | – |
| C14 | Optional | Scenario-dependent (Privacy) | – |
| C15 | Required | Cross-layer | Completed in Integration |
| C17 | Required | Layer-specific (Integration) | +C17.5 |
| C19 | Required | Layer-specific (Privacy) | +C19.5 Optional → required |
| C23 | Required | Layer-specific (Integration) | – |
| C24 | Required | Cross-layer | Optional → required; single-layer → cross-layer |
| C25 | Required | Layer-specific (Integration) | +C25.6, +C25.7 |
| C26 | Optional/conditional | Scenario-dependent | NEW |
| C27 | Required | Layer-specific (Access) | NEW |

A.16  Synthesis

The fifteen scenarios produced sixty-seven gap instances across fourteen distinct gaps. Three observations follow.

Layer diagrams under-determine deployment. The baseline architecture is internally consistent, yet scenario tracing found fourteen deficiencies. A layer diagram states which services carry cross-layer responsibility; it cannot state whether the corresponding capability is available in each layer. Three gaps arise from this conflation alone: trust resolution is a cross-layer responsibility but is unavailable where consent is evaluated (G4); FHIR services are cross-layer but not fully operationalised in the integration path (G1); and data minimisation is assigned to the Privacy and Policy layer but enforced at no gateway instance (G3). Scenario tracing exposes these because a trace must name which instance of a service handles each step — a question a diagram never has to answer.

The gaps require three different remedies. Twenty instances are placement gaps, where the component exists but is not available, mandatory or sufficiently replicated where required (G1, G4, G5, G7, G8); these are remedied by changing deployment. Forty-one are capability gaps, where a subcomponent implied by the parent component's stated responsibility was not operationalized (G2, G3, G6, G9, G13, G14); these are remedied within existing components. Six concern responsibilities no component owns (G10, G11, G12) and require new components. That capability gaps account for three-fifths of instances indicates a specification-to-implementation deficit rather than an error in the layer model, which is why the design survived review.

Scenario-set adequacy depends on condition diversity, not count. Gap frequency is strongly bimodal: three gaps account for 36 of 67 instances and surface under almost any scenario, while nine are each confined to a single scenario class and are invisible outside it. Fifteen scenarios spanning nine condition classes identified fourteen gaps; fifteen variations on one class would have identified three. Coverage must therefore be judged by the conditions represented — failure during operation, authorization change mid-episode, secondary use, cross-boundary invocation — rather than by scenario count.

This bound is itself limited. Scenario tracing did not find G14, and adding scenarios would not have found it. The R6 model specifies that candidate patient records are assessed for deterministic or probabilistic match before binding; the baseline provides binding through C19 but no matching capability. The registration scenario models a newly registering patient, for whom no candidate records exist and no matching is required. A scenario exercises the conditions its author anticipates, so condition diversity is bounded by what the analyst thinks to vary. Auditing requirement models against the derived component set does not share that bound, because it compares two artifacts rather than imagining a situation, and detected this case.

Threats to validity

A single analyst conducted the evaluation using scenarios derived from the same requirement set that produced the architecture; the re-validation in Table A-14 therefore demonstrates internal consistency rather than independent confirmation. Coverage of R16 is partial: the scenario set exercises failure detection and recovery but not the operator-facing behaviors the requirement specifies — metrics emission, retry-trace recording, alert signaling, incident correlation and escalation. The corresponding capability exists in C5.3 and C5.4, so this is a limitation of scenario coverage rather than of the architecture, although incident correlation as specified would additionally require the structured event classification identified as absent in G6.